DTCs are standardized codes stored by vehicle’s on-Board Diagnostics (OBD) system when it detects a malfunction.

Modern vehicles have an OBD-II system – a network of sensors and an Engine Control Module (ECM/ECU) that continuously monitors vehicle systems. When something falls outside normal parameters, it:

- Stores a DTC in memory.

- Illuminates the Check Engine Light (another warning light).

Code Structure:

Every DTC follows a 5-character format: B0123

| Position | Example | Meaning |
| --- | --- | --- |
| 1st-Letter | P | System: Powertrain,Body,Chassis, Unetwork |
| 2nd-Digit | 0 | 0 = SAE standard, 1 = Manufacturer-specific |
| 3rd-Digit | 1 | Subsystem (fuel, ignition, emissions, etc.) |
| 4th-5th-Digits | 23 | Specific fault within that subsystem |

How a Code Gets Set,

- A sensor sends a reading to ECU (e.g.., O2 sensor voltage)

- The ECU compares it against expected values.

- If it is out of range consistently (often over 2 drives cycles), a DTC is stored.

- The MIL (Malfunction Indicator Lamp) – the check engine light-turns on.

Reading the Codes.

You need an OBD-II scanner plugged into DLC (Data Link Connector) port, usually found under the dashboard on the driver’s side. Scanner range from:

- Basic Code Readers – Just reads/clears codes

- Professional scan tools – live data, freeze frame, bidirectional control.

- Smartphone adapters – paired with apps like Torque or Car Scanner.