UDS-Programming Session Sequence

Step 1 – Unlock security via service 0x27 SecurityAccess.

- Tell ECU application to unlock availability to transition to the programming session.

- This is a two-pass exchange- 1) Request new security level and get a seed. 2) Complete the unlock with a key derived from the seed. 3) Security level ID is not prescribed by ISO and is determined by the vehicle OEM.

Step 2- Enter Programming session via service 0x10 02 DiagnosticSessionControl – Programming Session

- Make ECU pass control from application to bootloader.

Step 3- Erase Flash Memory via service 0x31 01 FF 00 Routine Control – Start Routine – Erase Memory

- Tell bootloader to erase flash memory so that we have a clean flash part primed for programming.

Step 4 – Initiate download via servie 0x34 Request Download.

- Tell Bootloader address and size memory region we are about to program.